CCNA Practice Lab 01

VLANs + Trunking

Cisco Packet Tracer | Difficulty: 2/5 | Target Time: 30–45 minutes

# 1. Scenario

You are the junior network administrator for a small company. The company has two departments: Sales and IT. Two switches are used, and users from each department must be placed into separate VLANs. The switches are connected together, so the inter-switch link must carry traffic for both VLANs.

# 2. Topology

TRUNK
          ┌─────────────────┐
          │                 │
       ┌──┴──┐             ┌──┴──┐
       │ SW1 │──────────────│ SW2 │
       └──┬──┘              └──┬──┘
          │                    │
      ┌───┴───┐            ┌───┴───┐
      │       │            │       │
     PC1     PC2          PC3     PC4
    Sales   Sales          IT      IT

# 3. IP Addressing

| Device | VLAN | IP Address | Default Gateway |
| --- | --- | --- | --- |
| PC1 | 10 | 192.168.10.11/24 | None |
| PC2 | 10 | 192.168.10.12/24 | None |
| PC3 | 20 | 192.168.20.11/24 | None |
| PC4 | 20 | 192.168.20.12/24 | None |

Note: Do not configure a default gateway. Inter-VLAN routing is not part of this lab.

# 4. Port Assignments

| Switch | Interface | Connected Device |
| --- | --- | --- |
| SW1 | Fa0/1 | PC1 |
| SW1 | Fa0/2 | PC2 |
| SW1 | Gi0/1 | SW2 |
| SW2 | Fa0/1 | PC3 |
| SW2 | Fa0/2 | PC4 |

# 5. Objectives

Create VLAN 10 named SALES on both switches.

Create VLAN 20 named IT on both switches.

Configure the appropriate PC-facing interfaces as access ports.

Configure the SW1–SW2 link as an IEEE 802.1Q trunk.

Allow VLANs 10 and 20 on the trunk.

Verify VLAN membership, access-port assignments, trunk status, and MAC learning.

Test same-VLAN and different-VLAN connectivity.

Troubleshoot an intentionally incorrect VLAN assignment.

# 6. Lab Tasks

## Task 1 — Create VLANs

On both switches, create VLAN 10 named SALES and VLAN 20 named IT.

## Task 2 — Configure Access Ports

On SW1, place Fa0/1 and Fa0/2 in VLAN 10. On SW2, place Fa0/1 and Fa0/2 in VLAN 20.

## Task 3 — Configure the Trunk

Configure the SW1–SW2 link as a trunk. Explicitly allow VLANs 10 and 20. Do not use an unrestricted 'allow all VLANs' configuration.

## Task 4 — Configure End Devices

Assign the IP addresses from the addressing table to PC1–PC4. Do not configure default gateways.

## Task 5 — Verify

Verify VLANs, access-port assignments, trunk status, allowed VLANs, and MAC address learning.

## Task 6 — Test Connectivity

Test PC1 → PC2, PC3 → PC4, PC1 → PC3, and PC2 → PC4. Record whether each test succeeds or fails and explain why.

# 7. Verification Checklist

☐ VLAN 10 exists on SW1 and SW2.

☐ VLAN 20 exists on SW1 and SW2.

☐ PC1 and PC2 are assigned to VLAN 10.

☐ PC3 and PC4 are assigned to VLAN 20.

☐ The SW1–SW2 link is operating as a trunk.

☐ VLANs 10 and 20 are allowed on the trunk.

☐ Switches have learned the expected MAC addresses.

# 8. Useful Verification Commands

show vlan brief

show interfaces status

show interfaces trunk

show mac address-table

show running-config

show interfaces <interface>

# 9. Troubleshooting Challenge

After completing the lab successfully, intentionally create the following fault: change PC4's access port on SW2 so that PC4 is accidentally placed into VLAN 10 instead of VLAN 20.

Now test PC3 → PC4. Troubleshoot the problem without immediately comparing your configuration to a known solution. Use show commands to identify the fault, then fix it.

What command(s) did you use to identify the problem?

What was incorrectly configured?

What configuration change fixed the problem?

Why did the original configuration cause the connectivity problem?

# 10. Knowledge Check

1. Why does the SW1–SW2 link need to be a trunk?

Answer: _________________________________________________

________________________________________________________

2. What would happen if the SW1–SW2 link were configured as an access port in VLAN 10?

Answer: _________________________________________________

________________________________________________________

3. Can PC1 communicate with PC3 at this stage? Why or why not?

Answer: _________________________________________________

________________________________________________________

4. Does a trunk perform routing between VLAN 10 and VLAN 20?

Answer: _________________________________________________

________________________________________________________

5. What is the difference between an access port and a trunk port?

Answer: _________________________________________________

________________________________________________________

# 11. Solution / Instructor Reference

Try the entire lab before opening this section. The commands below are a reference, not a substitute for attempting the configuration yourself.

## VLAN Creation — SW1 and SW2

enable

configure terminal

vlan 10

name SALES

vlan 20

name IT

## SW1 Access Ports

interface range fa0/1-2

switchport mode access

switchport access vlan 10

## SW2 Access Ports

interface range fa0/1-2

switchport mode access

switchport access vlan 20

## Trunk — SW1 and SW2

interface gi0/1

switchport mode trunk

switchport trunk allowed vlan 10,20

## Expected Connectivity

| Test | Expected | Reason |
| --- | --- | --- |
| PC1 → PC2 | Success | Both devices are in VLAN 10. |
| PC3 → PC4 | Success | Both devices are in VLAN 20. |
| PC1 → PC3 | Fail | They are in different VLANs and no Layer-3 routing exists. |
| PC2 → PC4 | Fail | They are in different VLANs and no Layer-3 routing exists. |

## Troubleshooting Challenge — Expected Fault

PC4 should be connected to VLAN 20. If its access port is accidentally assigned to VLAN 10, PC3 and PC4 are placed into different VLANs even though both are intended to be in the IT department. Use 'show vlan brief' or 'show interfaces status' on SW2 to identify the incorrect VLAN assignment.

## Knowledge Check — Reference Answers

1. A trunk is required because the link between the switches must carry traffic for multiple VLANs.

2. If the link is an access port in VLAN 10, it will carry only VLAN 10 traffic as an access link; VLAN 20 traffic will not traverse it as a VLAN 20 trunk.

3. No. PC1 and PC3 are in different VLANs, and there is no Layer-3 device providing inter-VLAN routing.

4. No. A trunk transports multiple VLANs; it does not perform inter-VLAN routing.

5. An access port normally belongs to one VLAN and connects end devices. A trunk carries traffic for multiple VLANs between network devices using VLAN tagging.